|  |  |  | - ATX power supply connector pinout | - connector or cable wiring scheme |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 20 pin MOLEX 39-29-9202 connector at the motherboard |  | - v 1.x with 20 pin connector was widely used at PC motherboard. Replaced by v 2.x with 24 pin connector ATX specification includes not only Power Supply Unit, but also interface to case and motherboard. In addition to the old AT standard, ATX 2.0 has one extra voltage line available (+3.3V), a connector chain-lined to the single 20-pin and a power-on wire that allows Software to turn off the PSU. Several units (on hand) show pin-12 as Brown (not Blue), pin-18 as Blue (not White), and pin-8 as White (not Gray). Pin 9 (standby) goes from 0 to 5V and pin 14 goes from 0 to 3.7 when PSU switch is turned on. Shorting pin 14 (/PS_ON) to GND (COM) causes power supply to switch ON and PWR_OK to change to +5V. Pin; Name; Color; Description. 1; 3.3V; Orange; +3.3 VDC. 2; 3.3V; Orange; +3.3 VDC. 3; COM; Black; Ground. 4; 5V; Red; +5 VDC. 5; COM; Black; Ground. 6; 5V; Red; +5 VDC. 7; COM; Black; Ground. 8; PWR_OK; Gray; Power Ok is a status signal generated by the power supply to notify the computer that the DC operating voltages are within the ranges required for proper computer operation (+5 VDC when power is Ok). 9; 5VSB; Purple; +5 VDC Standby Voltage (max 10mA) 500mA or more typical. 10; 12V; Yellow; +12 VDC. 11; 3.3V; Orange; +3.3 VDC. 12; -12V; Blue; -12 VDC. 13; COM; Black; Ground. 14; /PS_ON; Green; Power Supply On (active low). Short this pin to GND to switch power supply ON, disconnect from GND to switch OFF.. 15; COM; Black; Ground. 16; COM; Black; Ground. 17; COM; Black; Ground. 18; -5V; White; -5 VDC (2002 v1.2 made optional, 2004 v2.01 removed from specification). 19; 5V; Red; +5 VDC. 20; 5V; Red; +5 VDC. /PSON activated by pressing and releasing the power button while the power supply is in standby mode. Activating /PS_ON turns on the power supply. Is there something need to be corrected? Edit this page! | 20 pin MOLEX 39-01-2200 connector at the cable |  |
|  |  |  |  |  | Related pinouts: ATX new power supply connector pinout v 2.x is a new ATX standard, use 24 pin connector. |  |

Source(s) of this and additional information: ATX Spec v2.03 at Platform Development Support, from Hardware Book, http://www.formfactors.org/developer%5Cspecs%5CPSU_DG_rev_1_1.pdf Table 21, I have a old (2001) computer w FSP235-60GT 235W power supply w 20 pin connector

Contributor: , Dave Johnson, Bart, Scot Norris, Christopher Chavasse, Jan Goldberg

| Pinouts.ru > ATX, BTX, WTX and other power supply cables connectors pinouts > Pinout of ATX power supply connector and layout of 20 pin MOLEX 39-29-9202 connector and 20 pin MOLEX 39-01-2200 connector. Is this document correct or incorrect? | Is it correct?: | should be correct |
| --- | --- | --- |
|  | 61 comments |  |
| Last updated 2010-08-26 22:35:18. Edit this page. |  |  |

[Discuss at the forum] [Back to index] [SUBMIT new pinout]

This page contain parts under Copyright © 2000-2010 by pinouts.ru team.
No portion of this webpage may be reproduced in any form without providing visible HTML link to pinouts.ru . Webmaster permission required in any other cases.
Efforts have been made to ensure this page is correct, but it is the responsibility of the user to verify the data is correct for their application.
536467 hits since March 4, 2005