MPS TECHNICAL BULLETIN	TB No 97/004

Date:	17 September 1997

Effective:		Immediately

Model:

9500 Series

Subject:

Setting Up Hard Disk

Setting Up Hard Disk

# Contents	Page

Contents Page

AMI BIOS SETTINGS

STANDARD

ADVANCED

CHIPSET

POWER MANAGEMENT SETUP

PCI/PNP SETUP

## AMI BIOS SETTINGS

To enter the BIOS Setup program;

- Reboot the system: upon startup a screen appears with a series of diagnostic checks.

- When “Hit <DEL> if you want to run SETUP” appears, press the <DEL> key to enter the BIOS setup program.  Enter the sections detailed below and set accordingly.

## STANDARD

Primary Master Hard Disk

| Primary Master Hard Disk |  |
| --- | --- |
| Type | User |
| LBA/Large Mode | Off |
| Block Mode | Off |
| 32 Bit Mode | Off |
| PIO Mode | Auto |

| Cylinders | Heads | WP | LZ | Sec | Size (WIB) |
| --- | --- | --- | --- | --- | --- |
| 1023 | 16 | 65535 | 1023 | 63 | 504 |

Primary Slave Hard Disk

| Primary Slave Hard Disk |  |
| --- | --- |
| Type | Not Installed |
| LBA/Large Mode | Off |
| Block Mode | Off |
| 32 Bit Mode | Off |
| PIO Mode | Auto |

Secondary Master Hard Disk Drive

| Secondary Master Hard Disk |  |
| --- | --- |
| Type | Not Installed |
| LBA/Large Mode | Off |
| Block Mode | Off |
| 32 Bit Mode | Off |
| PIO Mode | Auto |

Secondary Slave Hard Disk Drive

| Secondary Slave Hard Disk |  |
| --- | --- |
| Type | Not Installed |
| LBA/Large Mode | Off |
| Block Mode | Off |
| 32 Bit Mode | Off |
| PIO Mode | Auto |

FLOPPY A:	1.44 MB   3½"

FLOPPY B:		NOT INSTALLED

## ADVANCED

| Advanced Setup |  |
| --- | --- |
| 1st Boot Device | IDE-0 |
| 2nd Boot Device | Disabled |
| 3rd Boot Device | Disabled |
| 4th Boot Device | Disabled |
| Try other boot devices | No |
| S.M.A.R.T for hard disks | Disabled |
| Quick Boot | Enabled |
| Bootup Numlock | On |
| Floppy Drive swap | Disabled |
| Floppy Access Control | Normal |
| HDD Access Control | Normal |
| PS/2 Mouse Support | Disabled |
| Primary Display | VGA/EGA |
| Password Checking | Setup |
| Internal Cache | Enabled |
| External Cache | Enabled |
| System BIOS Cacheable | Disabled |
| Video Shadow C000, 32K | Disabled |
| Shadow C800, 16K | Disabled |
| Shadow CC00, 16K | Disabled |
| Shadow D000, 16K | Disabled |
| Shadow D400, 16K | Disabled |
| Shadow D800, 16K | Disabled |
| Shadow DC00, 16K | Disabled |

## CHIPSET

| Chipset Setup |  |
| --- | --- |
| DRAM lead off timing | 10/6/3 |
| Memory Address drive strength | Auto |
| SDRAM CAS Latency/RAS to CAS | 3/3 |
| Speculative lead off timing | Disabled |
| DRAM Page Idle Timeouts (HCLK’s) | 2 |
| Enhanced Paging | Enabled |
| DRAM refresh RAS cycles (HCLK’s) | 5 |
| Chipset NA# Asserted | Enabled |
| DRAM refresh rate | 15us |
| Suspend Refresh Type | CBR |
| Pipeline cache timing | Faster |
| 8-bit I/O Recovery time | 2 sysclk |
| 16-bit I/O Recovery time | 1 sysclk |
| USB Function | Disabled |
| USB Keyboard/Mouse support | Enabled |

## POWER MANAGEMENT SETUP

| Power Management Setup |  |
| --- | --- |
| Power Management/APM | Disabled |

## PCI/PNP SETUP

| PnP/PCI Setup |  |
| --- | --- |
| Plug and Play aware O/S | No |
| PCI Latency Timer (PCI Clocks) | 64 |
| PCI VGA Palette Snoop | Disabled |
| PCI IDE Bus Master | Disabled |
| Offboard PCI IDE Card | Auto |
| Offboard PCI IDE Primary IRQ | Disabled |
| Offboard PCI IDE Secondary IRQ | Disabled |
| Assign IRQ to PCI VGA Card | No |
| PCI Slot1 IRQ Priority | Auto |
| PCI Slot2 IRQ Priority | Auto |
| PCI Slot3 IRQ Priority | Auto |
| PCI Slot4 IRQ Priority | Auto |
| DMA Channel 0 | PnP |
| DMA Channel 1 | PnP |
| DMA Channel 3 | PnP |
| DMA Channel 5 | PnP |
| DMA Channel 6 | PnP |
| DMA Channel 7 | PnP |
| IRQ 3 | PCI/PnP |
| IRQ 4 | PCI/PnP |
| IRQ 5 | PCI/PnP |
| IRQ 7 | PCI/PnP |
| IRQ 9 | PCI/PnP |
| IRQ 10 | PCI/PnP |
| IRQ 11 | PCI/PnP |
| IRQ 12 | PCI/PnP |
| IRQ 14 | PCI/PnP |
| IRQ 15 | PCI/PnP |