- Intel Core i5-2500K CM8062300833803 (BX80623I52500K / BXC80623I52500K)

- Specifications

Top of Form

Compare with:

Bottom of Form

| General information |  |
| --- | --- |
| Type | CPU / Microprocessor |
| Market segment | Desktop |
| Family | Intel Core i5 |
| Model number ? | i5-2500K |
| CPU part numbers | CM8062300833803 is an OEM/tray microprocessor BX80623I52500K is a boxed processor (English version) BXC80623I52500K is a boxed processor (Chinese version) |
| Frequency (MHz) ? | 3300 |
| Turbo frequency (MHz) | 3700 |
| Clock multiplier ? | 33 |
| Package | 1155-land Flip-Chip Land Grid Array |
| Socket | Socket 1155 (Socket H2) |
| Size | 1.48" x 1.48" / 3.75cm x 3.75cm |
| Introduction date | Jan 9, 2011 |
| Price at introduction | $216 |
| S-spec numbers |  |
| ES/QS processors; Production processors. Part number; Q1J1; SR008. BX80623I52500K; +. BXC80623I52500K; +. CM8062300833803; +; +. |
| Architecture / Microarchitecture |  |
| Microarchitecture | Sandy Bridge |
| Processor core ? | Sandy Bridge |
| Manufacturing process | 0.032 micron High-K metal gate process |
| Data width | 64 bit |
| Number of cores | 4 |
| Floating Point Unit | Integrated |
| Level 1 cache size ? | 4 x 32 KB instruction caches 2 x 32 KB data caches |
| Level 2 cache size ? | 4 x 256 KB |
| Level 3 cache size | 6 MB shared cache |
| Cache latency | 4 (L1 cache) 11 (L2 cache) 25 (L3 cache) |
| Physical memory (GB) | 32 |
| Multiprocessing | Uniprocessor |
| Features | - MMX instruction set - SSE - SSE2 - SSE3 - Supplemental SSE3 - SSE4.1 ? - SSE4.2 ? - AES instructions - Advanced Vector Extensions - Extended Memory 64 technology (EM64T) ? - Execute disable bit ? - Turbo Boost technology 2.0 - Virtualization technology (VT-x) ? |
| Low power features | Enhanced SpeedStep technology ? |
| On-chip peripherals | - Dual-channel DDR3 memory controller - Direct Media Interface - HD graphics controller |
| Electrical/Thermal parameters |  |
| Maximum operating temperature (°C) ? | 72.6 |
| Thermal Design Power (W) ? | 95 |
| Notes on Intel CM8062300833803 |  |
| - Integrated graphics controller runs at 850 MHz, and up to 1100 MHz in Turbo mode - Memory controller supports DDR3-1066 and DDR3-1333 memory - The processor has unlocked clock multiplier |  |

- Pictures (5)

| Core i5-2500K CPU pictures |  |
| --- | --- |
| 2011-05-11 10:33:11 Posted by: gshv |  |
| Top view. S-spec number is SR008. The processor was manufactured in the first half of October 2010 (2 and half months before the launch). |  |
| 2011-05-11 10:30:52 Posted by: gshv |  |
| Bottom view |  |
| Intel Core i5-2500K Box |  |
| 2010-12-13 18:29:38 Posted by: Chrisch |  |
| Intel Core i5-2500K Box |  |
| 2010-12-13 18:29:15 Posted by: Chrisch |  |
| Intel Core i5-2500K Box |  |
| 2010-12-13 18:28:49 Posted by: Chrisch |  |
| Add picture |  |

- News

Store prices for new Core i5 and Core i7 processors

Jan 09, 2011: You may already know that today is an official launch day of Sandy Bridge microprocessors, dubbed by Intel as "2nd generation Core" CPUs. In contrast to majority of past launches, when it took from one to several days for microprocessors to materialize in stores, the new Sandy Bridge CPUs are already available in retail.

Top of Form

Bottom of Form

Intel launches new desktop and mobile CPUs

Jan 09, 2011: Earlier this week, at Consumer Electronics Show in Las Vegas, Intel announced 2nd generation Core microprocessors, based on just introduced Sandy Bridge micro-architecture. Today, Intel officially launched these processors, as well as a few models built on older Nehalem and Core technology. Intel's price list was updated with prices and brief specifications of all new CPUs.

Top of Form

Bottom of Form

Intel launches Sandy Bridge micro-architecture

Jan 06, 2011: On Monday, Intel announced new "2nd generation Core" desktop and mobile microprocessors, based on Sandy Bridge micro-architecture. The same day many hardware news sites published reviews of Sandy Bridge processors, and the universal conclusion was "yes, it's faster and better". Today Intel updated ARK database with details on all new models.

Top of Form

Bottom of Form

Prices and launch dates of desktop Sandy Bridge CPUs

Dec 02, 2010: While specifications of many Sandy Bridge desktop microprocessors were known for quite some time, since July of this year, prices and release dates of these models were unknown until last week. At the end of November Expreview revealed prices and launch dates of all Sandy Bridge-based desktop models. Expreview also published specifications of Pentium CPUs, that will be introduced in February and in the second quarter 2011.

Top of Form

Bottom of Form

Intel Sandy Bridge CPUs surface in Biostar CPU support list

Nov 16, 2010: Biostar published CPU support list for socket 1155 motherboards, which includes a number of upcoming Core i5 and Core i7 Sandy Bridge microprocessors. Processor frequency and Thermal Design Power characteristics in the published data match those from the earlier reports, that appeared months ago. The list references Core i5-2300 processor, that we first reported on three days ago.

Top of Form

Bottom of Form

Part numbers of desktop Sandy Bridge CPUs emerge on Intel website

Nov 13, 2010: Part numbers of OEM and boxed desktop microprocessors, based on upcoming Sandy Bridge micro-architecture, were published today on Intel's website. S-Spec numbers of these CPUs were also made public. This information confirms model numbers of upcoming Core i3, Core i5 and Core i7 processors, that were revealed in July - August of this year. Also, there will be one new quad-core model, Intel Core i5-2300.

Top of Form

Bottom of Form

- Benchmarks

| Top of Form Compare results with tested CPUs from one of the following families: Bottom of Form Top of Form To compare benchmark results with all other microprocessors please Bottom of Form |  |
| --- | --- |
| Tested processor |  |
| CPU ID: | 06A7h |
| CPU vendor string: | GenuineIntel |
| CPU name string: | Intel(R) Core(TM) i5-2500K CPU @ 3.30GHz |
| - Integer performance Part number; Result. Intel Core i7 i7-2600 CM8062300834302; 115.2%. Intel Core i7 i7-2600K CM8062300833908; 115%. Intel Core i5 i5-2500K CM8062300833803; 100%. Intel Core i5 i5-2500 CM8062300834203; 100%. Intel Core i7 I7-875K BV80605001905AM; 96%. Intel Core i7 I7-870 BV80605001905AI; 95.9%. Intel Core i5 i5-2400 CM8062300834106; 93.9%. Intel Core i7 I7-940 AT80601000921AA; 91.7%. |
| - Floating Point performance Part number; Result. Intel Core i7 I7-875K BV80605001905AM; 136.3%. Intel Core i7 I7-940 AT80601000921AA; 133%. Intel Core i7 I7-860 BV80605001908AK; 125%. Intel Core i7 I7-920 AT80601000741AA; 122.9%. Intel Core i5 i5-2500 CM8062300834203; 100.3%. Intel Core i5 i5-2500K CM8062300833803; 100%. AMD Phenom II X4 975 HDZ975FBK4DGM; 99.8%. AMD Phenom II X4 970 HDZ970FBK4DGR; 97.2%. AMD Phenom II X4 970 HDZ970FBK4DGM; 97.1%. Intel Core i5 i5-2400 CM8062300834106; 94.4%. AMD Phenom II X4 965 HDZ965FBK4DGI; 94.3%. |
| - MMX / SSE / SSE2 performance Part number; Result. Intel Core i7 I7-870 BV80605001905AI; 120.4%. Intel Core i7 I7-940 AT80601000921AA; 118.3%. Intel Core i7 I7-860 BV80605001908AK; 110.4%. Intel Core i7 I7-920 AT80601000741AA; 108.6%. Intel Core i5 i5-2500 CM8062300834203; 100.1%. Intel Core i5 i5-2500K CM8062300833803; 100%. Intel Core i5 i5-2400 CM8062300834106; 93.6%. AMD Phenom II X4 975 HDZ975FBK4DGM; 90.1%. AMD Phenom II X4 970 HDZ970FBK4DGR; 87.9%. AMD Phenom II X4 970 HDZ970FBK4DGM; 87.9%. AMD Phenom II X4 965 HDZ965FBK4DGM; 85.4%. |
| - Memory-intensive program performance Part number; Result. Intel Core i7 i7-2600 CM8062300834302; 104.5%. Intel Core i7 i7-2600K CM8062300833908; 104.5%. Intel Core i5 i5-2500K CM8062300833803; 100%. Intel Core i5 i5-2500 CM8062300834203; 99.6%. Intel Core i5 i5-2400 CM8062300834106; 94.9%. Intel Core i7 I7-870 BV80605001905AI; 91%. Intel Core i5 i5-2300 CM8062301061502; 88.7%. Intel Core i7 I7-875K BV80605001905AM; 88.2%. |

- CPU ID (1)

View / search public CPUID submissions | Check your CPU

NOTE: CPU ID information below was taken from one CPU and may include features that are not present in all different steppings of the Intel Core i5 i5-2500K CPU.

| Manufacturer: Intel. CPU Family:; Core i5. Processor Number:; i5-2500K. Frequency:; 3310 MHz. | Part number:; CM8062300833803. S-Spec Number:; SR008. Comment:. Submitted by:; CPU-World. |  |
| --- | --- | --- |
| General information |  |  |  |
| Vendor: | GenuineIntel |  |  |
| Processor name (BIOS): | Intel(R) Core(TM) i5-2500K CPU @ 3.30GHz |  |  |
| Cores: | 4 |  |  |
| Logical processors: | 4 |  |  |
| Processor type: | Original OEM Processor |  |  |
| CPUID signature: | 206A7 |  |  |
| Family: | 6 (06h) |  |  |
| Model: | 42 (02Ah) |  |  |
| Stepping: | 7 (07h) |  |  |
| TLB/Cache details: | 64-byte Prefetching Data TLB0: 2-MB or 4-MB pages, 4-way associative, 32 entries Data TLB: 4-KB Pages, 4-way set associative, 64 entries Instruction TLB: 4-KB Pages, 4-way set associative, 128 entries L2 TLB: 1-MB, 4-way set associative, 64-byte line size Shared 2nd-level TLB: 4 KB pages, 4-way set associative, 512 entries |  |  |

| Cache: | L1 data | L1 instruction | L2 | L3 |
| --- | --- | --- | --- | --- |
| Size: | 4 x 32 KB | 4 x 32 KB | 4 x 256 KB | 6 MB |
| Associativity: | 8-way set associative | 8-way set associative | 8-way set associative | 12-way set associative |
| Line size: | 64 bytes | 64 bytes | 64 bytes | 64 bytes |

| Instruction set extensions | Additonal instructions |
| --- | --- |
| MMX | CLFLUSH |
| SSE | CMOV |
| SSE2 | CMPXCHG16B |
| SSE3 | CMPXCHG8B |
| SSSE3 | FXSAVE/FXRSTORE |
| SSE4.1 | MONITOR/MWAIT |
| SSE4.2 | PCLMULDQ |
| AES instructions | POPCNT |
| AVX instructions | SYSENTER/SYSEXIT |
|  | XSAVE/XRESTORE states |
| Major features | Other features |
| On-chip Floating Point Unit | 36-bit page-size extensions |
| 64-bit / Intel 64 | 64-bit debug store |
| NX bit/XD-bit | Advanced programmable interrupt controller |
| Intel Virtualization | CPL qualified debug store |
| Turbo Boost | Debug store |
| Enhanced SpeedStep | Debugging extensions |
|  | Digital Thermal Sensor capability |
|  | LAHF/SAHF support in 64-bit mode |
|  | Machine check architecture |
|  | Machine check exception |
|  | Memory-type range registers |
|  | Model-specific registers |
|  | Page attribute table |
|  | Page global extension |
|  | Page-size extensions (4MB pages) |
|  | Pending break enable |
|  | Perfmon and Debug capability |
|  | Physical address extensions |
|  | Power Limit Notification capability |
|  | Process context identifiers |
|  | RDTSCP |
|  | Self-snoop |
|  | TSC rate is ensured to be invariant across all states |
|  | Thermal monitor |
|  | Thermal monitor 2 |
|  | Thermal monitor and software controlled clock facilities |
|  | Time stamp counter |
|  | Timestamp counter deadline |
|  | Virtual 8086-mode enhancements |
|  | xTPR Update Control |

See raw CPUID data

- Comments (1)

Add comment

- 'k' models

2011-02-28 19:03:13

Posted by: Abz Hussain

the K models are unlocked right?

Response: Yes, the core multiplier is unlocked on this and other "K" models.

Add comment

Last modified: 8 May 20