- Specifications

Dell™ OptiPlex™ 160L User's Guide

| Microprocessor Memory Computer Information Video Audio Expansion Bus Drives | Connectors Key Combinations Controls and Lights Power Physical Environmental Network |
| --- | --- |

| Microprocessor |  |
| --- | --- |
| Microprocessor type | Intel® Pentium® 4 and Celeron®; design provides for future Dell-supported upgrades. |
| Level 1 (L1) cache | 8 KB |
| Level 2 (L2) cache | 128-KB or 512-KB (depending on your computer configuration) pipelined-burst, eight-way set associative, write-back SRAM |

| Memory |  |
| --- | --- |
| Type | 266- and 333-MHz DDR SDRAM |
| Memory connectors | Two |
| Memory capacities | 128-, 256-, 512-MB, or 1-GB non-ECC |
| Minimum memory | 128 MB NOTE: Between 1 and 64 MB of system memory may be allocated to support graphics, depending on system memory size and other factors. |
| Maximum memory | 2 GB |
| BIOS address | F0000h |

| Computer Information |  |
| --- | --- |
| Chip set | Intel 845GV |
| Data bus width | 64 bits |
| Address bus width | 32 bits |
| DMA channels | Seven |
| Interrupt levels | 24 |
| BIOS chip | 4-Mb |

| Network |  |
| --- | --- |
| Network Controller | Integrated 10/100 Ethernet |

| Video |  |
| --- | --- |
| Type | Integrated Intel 3D Extreme Graphics |

| Audio |  |
| --- | --- |
| Type | AC97, Sound Blaster emulation |
| Stereo conversion | 16-bit analog-to-digital; 20-bit digital-to-analog |

| Expansion Bus |  |
| --- | --- |
| Bus type | PCI |
| Bus speed | 33 MHz |
| PCI |  |
| connector | Three |
| connector size | 120 pins |
| connector data width (maximum) | 32 bits |
| bus protocols | 4x/2x modes at 1.5 V |

| Drives |  |
| --- | --- |
| Internally accessible drives | Two bays for 1-inch-high IDE hard drives |
| Externally accessible drives | One bay for a floppy drive Two bays for CD/DVD drives |

| Connectors |  |
| --- | --- |
| External connectors: |  |
| Serial | 9-pin connector; 16550C-compatible |
| Parallel | 25-hole connector (bidirectional) |
| Video | 15-hole connector |
| Network adapter | RJ45 connector |
| PS/2 (keyboard and mouse) | 6-pin mini-DIN |
| USB | Two front-panel and four back-panel USB 2.0–compliant connectors |
| Audio | Three connectors for line-in, line-out, and microphone; one front-panel connector for headphones |
| System board connectors: |  |
| Primary IDE drive | 40-pin connector on PCI local bus |
| Secondary IDE drive | 40-pin connector on PCI local bus |
| Floppy drive | 34-pin connector |
| CD drive audio interface | 4-pin connector |
| Fan | 3-pin connector |

| Key Combinations |  |
| --- | --- |
| <Ctrl><Alt><\> | Toggles microprocessor speeds on 101-key keyboard (in MS-DOS® real mode only) |
| <Ctrl><Alt><#> | Toggles microprocessor speeds on 102-key keyboard (in MS-DOS real mode only) |
| <F2> or <Ctrl><Alt><Enter> | Starts embedded system setup (during start-up only) |
| <F3> | Automatically starts the computer from the network environment specified by the remote boot environment (PXE) rather than from one of the devices in the system setup Boot Sequence option (during start-up only) |
| <F12> or <Ctrl><Alt><F8> | Displays a one-time boot menu that lets the user enter a device for a single boot (during start-up only) |
| <Ctrl><Alt><F10> | Launches the utility partition (if installed) during computer start-up |

| Controls and Lights |  |
| --- | --- |
| Power control | Push button |
| Hard-drive access light | Green |
| Link integrity light (on integrated network adapter) | Green light for 10-Mb operation; orange light for 100-Mb operation |
| Activity light (on integrated network adapter) | Yellow blinking light |
| Diagnostic lights | Four lights on the back panel |

| Power |  |
| --- | --- |
| DC power supply: |  |
| Wattage | 200 W or 250 W |
| Heat dissipation | 682 or 853 BTU/hr (fully loaded computer without monitor) |
| Voltage | manual selection voltage power supply—100 to 120V at 60 Hz; 200 to 240V at 50Hz |
| Backup battery | 3-V CR2032 lithium coin cell |

| Physical |  |
| --- | --- |
| Height x Width x Depth | 36.8 x 18.4 x 42.6 cm (14.5 x 7.25 x 16.75 inches) |
| Weight | 10.4 kg (23 lb) |

| Environmental |  |
| --- | --- |
| Temperature: |  |
| Operating | 10° to 35°C (50° to 95°F) NOTE: At 35°C (95°F), the maximum operating altitude is 914 m (3000 ft). |
| Storage | –40° to 65°C (–40° to 149°F) |
| Relative humidity | 20% to 80% (noncondensing) |
| Maximum vibration: |  |
| Operating | 0.25 G at 3 to 200 Hz |
| Storage | 2.20 Grms at 10 to 500 Hz |
| Maximum shock: |  |
| Nonoperating (half-sine pulse) | 105 G, 2 ms |
| Nonoperating (faired-square wave) | 32 G with a velocity change of 596.9 cm/sec (235 inches/sec) |
| Altitude: |  |
| Operating | –15.2 to 3048 m (–50 to 10,000 ft) NOTE: At 35°C (95°F), the maximum operating altitude is 914 m (3000 ft). |
| Storage | –15.2 to 10,668 m (–50 to 35,000 ft) |