Support; Top of Form Bottom of Form.

|  |  | Specifications Show Part Numbers Hide Part Numbers 105553, 4006105R Following are the specifications for the FIC KTBC51G motherboard. Feature; Specification. Form Factor;  uBTX 4-layer  Dimensions: 10.4 × 10.5 inches. CPU Support; Package: Socket 939 Processors: - Athlon™ 64 3000+ ~ 4000+ - Athlon FX FX 55 ~ FX59 - Athlon 64 X2 4200+ ~ 4800+. System Bus Support; 2000 Mhz. Memory Support;  Four DDR DIMM slots  DDR 333/400 dual channel  Max Memory 4 GB. MCH (northbridge); C51G. ICH (southbridge); MCP51G. Firmware Hub/SPI; 4 MB. LAN Support; Integrated NVIDIA network controller with AC131 10/100 PHY. Integrated Audio;  Interface: Azalia  Model: Realtek ALC 880. IEEE-1394a Controller; VIA 6307L. Serial Input/Output (SIO); SMSC SCH5017. Add-in Card Slots;  One PCI Express ×16 (graphics interface)  One PCI Express ×1  Two PCI conventional. Rear panel I/O Ports;  Four USB 2.0  Two PS/2 keyboard/mouse  One parallel  One serial  One VGA monitor  Six Analog audio jacks  One RJ-45 LAN connector. Internal Headers;  Two 2-port USB 2.0 headers (2 × 5)  Front panel audio header (mic / headphone)  Two IEEE-1394a Front Panel Headers  Front panel header (2 × 5 supports power LED, power switch, hard drive LED and reset switch)  Floppy disk drive connector  Primary IDE connector  Secondary IDE connector  Two Serial ATA connectors  Clear CMOS jumper  Chassis Intrusion Header  Main Power Connector 24-pin  Auxiliary 12V CPU power connector 4-pin  CPU Fan; 4-pin, controlled  PWR Fan (Closed Loop / Always On) 3-pin  SYS Fan (Closed Loop / Always On) 4-pin; controlled. |  |  |
| --- | --- | --- | --- | --- |

Top of Form

Bottom of Form