|  |  | Specifications Part Number: 1008842Gateway GT4016 Media Center Computer Model: GT4016 Following are the specifications for the Gateway® GT4016 desktop computer. Feature; Specification. Processor; AMD Athlon™ 64 processor 3700+ - 2.2 GHz - 2000 MHz FSB - 1 MB L2 cache - 939 pin package. Chipset; C51G chipset. Firmware Hub (FWH); 4 MB. Memory;  1024 DDR dual channel memory  Four DIMM slots support DDR 333/400 dual channel modules  Expandable to 4 GB. Hard Disk Drive; 200 GB hard drive (7,200 RPM, 2 MB cache). Optical Drive; DVD +/- RW 16x multiformat double layer optical drive. Drive Bays;  Two external 5.25-inch  Two external 3.5-inch  Two internal 3.5-inch (hard drive). Network Support; Integrated NVIDIA network controller with AC131 10/100 PHY. Video; Integrated NVIDIA C51G video chipset. Discrete Graphics (Optional); Type: PCI-Express ×16. Audio; Integrated Realtek ALC 880, 8 channel (7.1) high definition audio. Add-in Card Slots;  One PCI Express ×16  One PCI Express ×1  Two PCI conventional. Media Reader; 9-in-1 Digital Media Manager supporting the following: - CompactFlash Card - IBM MicroDrive - Memory Stick - Memory Stick Pro - SmartMedia Card - Secure Digital(SD)/MMC - Compact Flash - Multimedia Card - xD-Picture Card - USB 1.1 or later. Ports;  Rear External - Four USB - Two PS/2 (keyboard and mouse) - One Parallel - One Serial - One VGA - Six Analog Audio jacks - RJ-45 LAN  Front External - Three USB (one in media manager, two on front ports panel) - Two IEEE 1394a 6-pin - Two audio 1/8-inch mini-jacks - Headphone (green) - Microphone (red). Power Supply; 300 watt. Temperature;  Operating: 5° C to 35° C (41° F to 95° F)  Non-operating: -20° C to 60° C (-4° F to 140° F)  Storage: -20° C to 60° C (-4° F to 140° F). Humidity;  Operating: Wet bulb method, not to exceed 33° C (91.4° F)  Non-operating: 95% relative (non-condensing) at 30° C (86° F). Maximum Vibration;  Operating: 0.2 g, 5 Hz - 300 Hz  Non-operating: 0.5 g, 5 Hz - 300 Hz. Maximum Shock;  Operating: 0.5 g, 11 ms, ½ sine  Non-operating: 40g, 11ms, ½ sine. Altitude; Operating: 10,000 ft max; maximum Temperature is derated 0.5° C for every 305 m (1000 ft) above sea level. Physical Dimensions (H × W × D); 15.4 × 7.3 × 17.9 inches. Weight (Unpackaged); 23.5 lbs. (varies with configuration). |  |  |
| --- | --- | --- | --- | --- |