How To Flash The BIOS By Yourself ?

|  | Summary If your system currently operate smoothly and there is no problem happened, your system is not suggested to do the BIOS revise, since improper operation or wrong BIOS code update might cause the system unable to start again. So we recommend to flash the BIOS only when the problem needs to be solved by revised BIOS and you need to read the detail procedures stated below. |
| --- | --- |
|  | The requirement of flashing BIOS To flash the BIOS, you neeed the following program and BIOS code: - Latest version BIOS code - BIOS flash program All of these two items can be downloaded from BIOS area of our web site. Check the BIOS date code to see if they are the same as before. If so, it's not necessary to flash it since BIOS does not have newer revision. Before flashing it, make sure the BIOS manufacturer and the model number of your board. There are two kinds of BIOS Flash program as below. - AWDfl628.exe: a specified BIOS flash program for AWARD BIOS. - AMIfl807.exe :a specified BIOS flash program for AMI BIOS. If your mainboard use AMI BIOS will be required the BIOS flash program of AMIfl807.exe. Otherwise, you need AWDfl628.exe flash program. You can find out the source from BIOS area of our web site easily. |

|  | The procedure of flashing BIOS Below is an example of using AMI flash program to flash the BIOS. - Under Dos mode C:(or A:), key in the name of BIOS flash program that downloaded from our web site. e.g.:C:\AMIFL807<Enter> - Being executed, the BIOS flash program will request you to give the file name and path for back up the existing BIOS. e.g.:C:\BACKUP.ROM<Enter> - After BIOS backup, key in the file name and path which to be flash programmed. e.g.:C:\0605s<Enter> - Restart the system. |
| --- | --- |
|  | Remark If you followed and done all the flash programming procedures above, but still fail to revise the BIOS of your board, please contact your system engineer or contact us by E-mail. |