//        General notes:

If you are upgrading from a previous version of Microtek Scan

Module, be sure to delete all the files from \WINDOWS\TWAIN

directory prior to installation of the new version.  Also, modify

your WIN.INI file in the Windows directory and remove the

[Microtek Scanner] section from it.  This section looks something

like the following.  Be sure to remove all lines and save the

changes.

[MICROTEK SCANNER]

Interface=SCSI

PortAddr=D400

SCSI ID=6

ScanType=ScanMaker IISP

// ------------------------------------------------------------------------

//  Scanner Return Status

// ------------------------------------------------------------------------

( 0)        // Scanner has partial communication.  You may need to change

your cable or install a terminator on the back of the

scanner.  Terminator is a device commonly used on the Macintosh

computers to filter the noise out. Sometimes, PC systems

also require such device.  You can obtain a terminator from

your local dealer.  Place the terminator between the cable and

the scanner.

If you are upgrading from a previous version of Microtek Scan

Module, be sure to delete all the files from \WINDOWS\TWAIN

directory prior to installation of the new version.  Also, modify

your WIN.INI file in the Windows directory and remove the

[Microtek Scanner] section from it.  This section looks something

like the following.  Be sure to remove all lines and save the

changes.

[MICROTEK SCANNER]

Interface=SCSI

PortAddr=D400

SCSI ID=6

ScanType=ScanMaker IISP

(-1)        // Scanner link failure message. Delete the file

SCAN.PAR and SCAN.MTA from \WINDOWS\TWAIN directory and try

again.

(-2)        // Scanner command error message.

(-3)        // Scanner hardware error message.

(-4)        // Scanner setting error message.

(-5)        // Scanner Miscellaneous error message.

(-6)        // Scanner Undefine error message.

(-100)      // Scanner SCSI command or data format error.  Delete the file

SCAN.PAR and SCAN.MTA from \WINDOWS\TWAIN directory and try

again.

If you are upgrading from a previous version of Microtek Scan

Module, be sure to delete all the files from \WINDOWS\TWAIN

directory prior to installation of the new version.  Also, modify

your WIN.INI file in the Windows directory and remove the

[Microtek Scanner] section from it.  This section looks something

like the following.  Be sure to remove all lines and save the

changes.

[MICROTEK SCANNER]

Interface=SCSI

PortAddr=D400

SCSI ID=6

ScanType=ScanMaker IISP

(-101)      // Too many lines requested in a READ SCANNED DATA command. Delete the file

SCAN.PAR and SCAN.MTA from \WINDOWS\TWAIN directory and try

again.

(-200)      // Scanner CPU RAM failure.

(-201)      // Scanner system RAM failure.

(-202)      // Pin pon buffer RAM failure.

(-203)      // Scanner CCD DC offset failure.

(-204)      // Scanning lamp or image sensor circuit failure.

(-205)      // Home position sensor or filter sensor failure.

(-206)	// Auto	document feeder	paper jam or roller failure.

(-207)	// External power adapter failure.  If using a ScanMaker IIG

or IISP, you need to have an external power adapter

connected to the scanner.  If the power supply is not

operational, you will get this error message.

(-300)      // Illegal grain setting.

(-301)      // Illegal resolution setting.

(-302)      // Illegal scanning frame coordinate.

(-303)      // Illegal window coordinate or number of windows.

(-304)      // Illegal contrast setting.

(-305)      // Illegal paper length setting.

(-306)      // Illegal shadow-midtone-hilight setting.

(-307)      // Illegal exposure time setting.

(-308)      // Color filter command when ADF scanning.

(-309)      // No paper when ADF scanning.

(-310)      // Illegal gamma table setting.

(-311)      // Illegal brightness setting.

(-312)      // Illegal data bits per pixel setting.

(-400)      // Scanner is in busy state.

(-401)      // Scanner safety door sensor failure.

(-501)      // Scanner has been reserved.

// ------------------------------------------------------------------------

//  Device Driver Return Status

// ------------------------------------------------------------------------

(1)         // Scanner is in ready state.  Microtek Scanmaker II/IIXE

firmware (EPROM) may be too old.  Call Microtek sales lines

for an upgrade.  You need to have firmware version 5.07 or

higher to use Microtek Scan Module version 1.45 or higher.

You can find out what version of firmware your scanner has

by checking your scanner box.  Next to the serial number

and the bar code printed on the box, you should be able to

see the letters "F/W".  The number next to F/W designates

your firmware version number.

(-1000)     // No response from scanner.

(-1001)     // Scanner is in scanning state.

(-1002)     // Scanner busy.

(-1003)     // Abort to get image data by application.

(-1004)     // Can't find scanner.  See the troubleshooting section of your

manual on how to fix this problem.  This error usually appears

when there is a memory address conflict between the Microtek

scanner card and another card in the system.  You may also

have problems if you have a memory manager such as EMM386.EXE

or QEMM.SYS.  To resolve this problem, you need to exclude

the range that your PCZ card is using from memory so no other

program can use that.  The setup program normally does this

for you automatically but you may want to read through the

following discussion just to make sure.  Add a new parameter

to your EMM386.EXE line or to your QEMM.SYS line that reads,

x=d400-d7ff if your scanner card is set to the default of d400.

The new line now should read,

DEVICE=C:\DOS\EMM386.EXE <other parameters here>   x=d400-d7ff

If you do not have EMM386.EXE or QEMM.SYS or 386MAX installed,

add the above line to your CONFIG.SYS file.

Then add a new line to the file SYSTEM.INI in the [386ENH] section

that reads, EMMExclude=d400-d7ff and reboot the computer.

If you are using another address setting, you need to change

these numbers.  For example, if you are using address d800, the

parameter changes to d800-dcff.  Microtek MS-PCZ cards use 16KB of

memory.  On some computers, you need to exclude 32KB of memory by

putting d400-dcff for memory address d400.

We have also exprienced that some 486 computers do not work at

dxxx ranges.  For these computers, use addresses E000 or E400

and exclude e000-e3ff or e400-e7ff depending on which address

you are using from your CONFIG.SYS and SYSTEM.INI files.

The Microsoft Diagnostics MSD.EXE when executed from a DOS prompt

can help you find a suitable address setting for the MS-PCZ card.

Type MSD, press Enter and select "M" for memory.  A menu

showing the status of various memory address locations will be

displayed on your screen.  Select a suitable address range which

has all black boxes (no dots or x in them.)  You can also

select an address with "X" in it and exclude it from memory

in your CONFIG.SYS and SYSTEM.INI's [386ENH] section.  See the

trouble-shooting notes or the README.TXT file in the Microtek

Calibrator folder for more details.

(-1005)     // Scanner type error.

(-1006)     // Device type error.

(-1007)     // Not enough memory.

(-1008)     // Undefine error codes.

(-1009)     // Can't find index error.

// ------------------------------------------------------------------------

//  MPHASE.DLL Return Status

// ------------------------------------------------------------------------

(-2000)     // SCSI host buffer not ready.

(-2001)     // SCSI control status error.

(-2002)     // SCSI unknown error codes.

// ------------------------------------------------------------------------

//  MSCSID.DLL, MPCYD.DLL, MLCARDD.DLL Return Status

// ------------------------------------------------------------------------

(-3000)     // SCSI card time out.

(-3001)     // PCY card time out.

(-3002)     // L card time out.

(-3100)     // SCSI Load library failure.  You may need to quit Windows,

go to the \WINDOWS\TWAIN directory and delete the files

SCAN.PAR and SCAN.MTA.  Turn the computer off and back on

to reset the interface card (warm boot or the reset button

is not sufficient), turn the scanner off and back on and try

again.  If the problem persists, change the memory address

on the interface card to e000 or e400 and exclude a 16KB range

from your CONFIG.SYS and SYSTEM.INI files.  If you are not

sure how to do this, refer to README.TXT or error -1004 section

above for a brief discussion.

If you are upgrading from a previous version of Microtek Scan

Module, be sure to delete all the files from \WINDOWS\TWAIN

directory prior to installation of the new version.  Also, modify

your WIN.INI file in the Windows directory and remove the

[Microtek Scanner] section from it.  This section looks something

like the following.  Be sure to remove all lines and save the

changes.

[MICROTEK SCANNER]

Interface=SCSI

PortAddr=D400

SCSI ID=6

ScanType=ScanMaker IISP

(-3101)     // PCY Load library failure.

(-3102)     // LCARD Load library failure.

// ------------------------------------------------------------------------

//  MSCANMGR.DLL Return Status

// ------------------------------------------------------------------------

(-4000)     // Scanner manager time out.

(-4001)     // Load library failure.